3.5 inch ESP32 module

List

Product description	 3

Features	3

Product parameters	 4

Interface Description	 5

Product Size	 6

3. 5寸  ESP32-035                      第 2   页 共 5   页

# Product description

This LCD module uses ESP32-WROOM-32 module as the main control, the main control  is  a  dual-core   MCU,  integrated  WI-FI  and   Bluetooth  functions,  the  main frequency  can  reach  240MHz,  520KB  SRAM,  448KB  ROM,  Flash  size  is  4MB,  The display  resolution  is  320X480,  resistive  touch.  The   module   includes  LCD  display screen,  backlight  control  circuit, touch  screen control circuit,  speaker drive circuit, photosensitive circuit and RGB-LED control circuit. TF card interface, serial interface, temperature and humidity sensor interface (DHT11 interface) and reserved IO port interface, this module supports development in arduino IDE, ESP IDE, Micropython and Mixly.

# Features

# ●3. 5-inch color screen, support 16 BIT RGB 65K color display, display rich

colors

●320X480 resolution

●The sample program has been programmed in the factory and can be plugged in

●With TF card slot for easy expansion storage

●Provide  arduino  library  functions  and  sample  programs  to  facilitate rapid secondary development

●Support one-click download program

●Military-grade process standards, long-term stable work

●Supports  lithium  battery  power  supply,  supports  both  charging  and discharging, and has over charging and over discharging protection

3. 5寸  ESP32-035                     第 3   页 共 5   页

# Product parameters

| Name | Describe | Remark |
| --- | --- | --- |
| Display color | RGB 65K color |  |
| SKU | without touch：ESP32-035 |  |
| Size | 3. 5 inch |  |
| Type | TFT |  |
| Driver chip | ST7796 |  |
| Resolution | 320*480(Pixel) |  |
| Effective display area | 55.2*85(mm) |  |
| Module size | 55.34*101.5(mm) |  |
| View | >60 ° |  |
| Operating temperature | -20℃~70℃ |  |
| storage temperature | -30℃~80℃ |  |
| Operating Voltage | 5V |  |
| Power consumption | About 100mA |  |
| Product weight | About 40g |  |

3. 5寸  ESP32-035                     第 4   页 共 5   页

# Interface Description

3.5寸  ESP32-035                     第 5   页 共 5   页